Design and Implementation of TARF: A Trust-Aware Routing Framework for WSNs

Aim:-

The most fundamental functions of WSNs. In a data collection task, a sensor node sends its sampled data to a remote base station with the aid of other intermediate nodes,

Abstract:-

The multi-hop routing in wireless sensor networks (WSNs) offers little protection against identity deception through replaying routing information. An adversary can exploit this defect to launch various harmful or even devastating attacks against the routing protocols, including sinkhole attacks, wormhole attacks and Sybil attacks. The situation is further aggravated by mobile and harsh network conditions. Traditional cryptographic techniques or efforts at developing trust-aware routing protocols do not effectively address this severe problem. To secure the WSNs against adversaries misdirecting the multi-hop routing, we have designed and implemented TARF, a robust trust-aware routing framework for dynamic WSNs. Without tight time synchronization or known geographic information, TARF provides trustworthy and energy-efficient route. Most importantly, TARF proves effective against those harmful attacks developed out of identity deception; the resilience of TARF is verified through extensive evaluation with both simulation and empirical experiments on large-scale WSNs under various scenarios including mobile and RF-shielding network conditions.

Existing System:

Wireless sensor networks (WSNs)  are ideal candidates for applications to report detected events of interest, such as military surveillance and forest fire monitoring. A WSN comprises battery-powered senor nodes with extremely limited processing capabilities With a narrow radio communication range, a sensor node wirelessly sends messages to a base station via a multi-hop path. However, the multi-hop routing of WSNs often becomes the target of malicious attacks. An attacker may tamper nodes physically, create traffic collision with seemingly valid transmission, drop or misdirect messages in routes, or jam the communication channel by creating radio interference.

Proposed System:

In this  proposed system focuses on the kind of attacks in which adversaries misdirect network traffic by identity deception through replaying routing information. Based on identity deception, the adversary is capable of launching harmful and hard-to-detect attacks against routing, such as selective forwarding, wormhole attacks, sinkhole attacks and Sybil attacks.

Modules:

- Node Creation

- Nodeconnection

- Data Transmissin (node to node)

- Hacking & security

System Requirement:

Hardware Requirements

- System		: Pentium IV 2.4 GHz

- Hard disk		: 40 GB

- Monitor		: 15 VGA colour

- Mouse		: Logitech.

- Ram			: 256 MB

- Keyboard		: 110 keys enhanced.

Software Requirements

- Operating system 		:- Windows XP Professional

- Programming language  : C# .NET

- Tools				: Microsoft Visual Studio 2010

- Back End			: Sql Server