AN AUTOMATIC WAVELET-BASED NONLINEAR IMAGE

ENHANCEMENT TECHNIQUE FOR AERIAL IMAGERY

SYNOPSIS

Aerial images captured from aircrafts, spacecrafts, or satellites usually suffer from lack of clarity, since the atmosphere enclosing Earth has effects upon the images such as turbidity caused by haze, fog, clouds or heavy rain. The visibility of such aerial images may decrease drastically and sometimes the conditions at which the images are taken may only lead to near zero visibility even for the human eyes. Even

though human observers may not see much than smoke, there may exist useful information in those images taken under such poor conditions.

Captured images are usually not the same as what we see in a real world scene, and are generally a poor rendition of it. High dynamic range of the real life scenes and the limited dynamic range of imaging devices results in images with locally poor contrast. Human Visual System (HVS) deals with the high dynamic range scenes by compressing the dynamic range and adapting locally to each part of the scene. There are some exceptions such as turbid (e.g. fog, heavy rain or snow) imaging conditions under which acquired images and the direct observation possess a close parity [1]. The extreme narrow dynamic range of such scenes leads to extreme low contrast in the acquired images To deal with the problems caused by the limited dynamic range of the imaging devices, many image processing algorithms have been developed  .These algorithms also provide contrast enhancement to some extent.

Recently we have developed a wavelet-based dynamic range compression (WDRC) algorithm to improve the visual quality of digital images of high dynamic range scenes with non-uniform lighting conditions .The WDRC algorithm is modified  by introducing an histogram adjustment and non-linear color restoration process so that it provides color constancy and deals with “pathological” scenes having very strong spectral characteristics in a single band. The fast image enhancement algorithm which provides dynamic range compression preserving the local contrast and tonal rendition is a very good candidate in aerial imagery applications such as image interpretation for defense and security tasks.